Katedra Systemów i Sieci Radiokomunikacyjnych Zespół projektowy: ID-629; 1. Bartosz Chełmiński - kierownik 2. Cyprian Bilski 3. Jakub Brzozowski. Opiekun:; dr inż. Piotr Rajchowski. Klient:; dr inż Piotr Rajchowski. Data zakończenia:. Słowa kluczowe:; system bezprzewodowy, transmisja dźwięku, gitara elektryczna. TEMAT PROJEKTU: Układ do bezprzewodowego przesyłania dźwięku z gitary elektrycznej. CELE I ZAKRES PROJEKTU: Celem projektu jest wykonanie Układu bezprzewodowego do przesyłania sygnału z gitary elektrycznej, nadający się do zastosowania przez profesjonalistów w warunkach koncertowych, co narzuca konieczność przesłania dźwięku wysokiej jakości. Parametry dźwięku : - Częstotliwość próbkowania 44.1 kHz - Rozdzielczość próbkowania 16 Bitów Założenia dot. łącza bezprzewodowego: - Opóźnienia poniżej 10 ms - Zasięg komunikacji urządzenia co najmniej 20 m. OSIĄGNIĘTE REZULTATY: - Zakupiono oraz dobrano odpowiednie moduły niezbędne do realizacji projektu; - Połączono moduły i wstępnie skonfigurowano kodek audio jako DAC dla układu odbiorczego oraz moduł radiowy zarówno dla nadawczego jak i odbiorczego; - Wykonano pierwszy przesył sygnału cyfrowego w celu testu łącza radiowego.. CECHY CHARAKTERYSTYCZNE ROZWIĄZANIA, KIERUNKI DALSZYCH PRAC: Do realizacji projektu zdecydowano się na poniższe komponenty: - Moduły WaveShare Audio Board wykorzystujący kodek WM 8960 - Mikrokontrolery ESP32 S3 obsługujące protokół ESP NOW korzystający z WiFi - Anteny dipolowe ze względu na dookólną charakterystykę W przyszłości planowane jest: - Wyeliminowanie nieciągłości sygnału oraz zapewnienie jego odpowiedniej jakości. - Dla układu nadawczego wykonanie bloku wejściowego (bufor + filtr antyaliasingowy) oraz dodanie i skonfigurowanie kodeka audio jako ADC. - W przypadku za wysokiego opóźnienia możliwa modyfikacja architektury układów o jednostkę centralną w postaci mikrokontrolera STM32F7. - Dodanie układu zasilania ogniwem litowo-jonowym, bądź litowo-polimerowym. - Wykonanie obudowy wraz z ekranowaniem zakłóceń z otoczenia. - Zrealizowanie wielodostępu, żeby systemy dzieliły się dostępnymi kanałami WiFi. - Test systemu z rzeczywistą gitarą i wzmacniaczem..

Department of Radiocommunication Systems and Networks Project team: ID-629; 1. Bartosz Chełmiński - kierownik 2. Cyprian Bilski 3. Jakub Brzozowski. Supervisor:; dr inż. Piotr Rajchowski. Client:; dr inż Piotr Rajchowski. Date:. Key words:; wireless system, audio streaming, electric guitar. PROJECT TITLE: Wireless system for transmitting an electric guitar signal. OBJECTIVES AND SCOPE: The objective of the project is to develop a wireless system for transmitting the signal from an electric guitar, suitable for use by professionals in live concert conditions, which requires the transmission of high-quality audio. Audio parameters: - Sampling frequency: 44.1 kHz - Sampling resolution: 16 bits Wireless link requirements: - Latency below 10 ms - Communication range of at least 20 m. RESULTS: - Appropriate modules necessary for the implementation of the project were purchased and selected. - The modules were connected and initially configured, with the audio codec set up as a DAC for the receiving system and the radio module configured for both the transmitting and receiving units. - The first digital signal transmission was performed to test the radio link.. MAIN FEATURES, FUTURE WORKS: The following components were selected for the project implementation: - WaveShare Audio Board modules using the WM8960 codec - ESP32-S3 microcontrollers supporting the ESP-NOW protocol based on Wi-Fi - Dipole antennas due to their omnidirectional radiation pattern Planned future work includes: - Eliminating signal discontinuities and ensuring adequate audio quality - For the transmitting system, implementing an input stage (buffer + anti-aliasing filter) and adding and configuring the audio codec as an ADC - In the case of excessive latency, a possible modification of the system architecture to include a central processing unit in the form of an STM32F7 microcontroller - Adding a power supply based on a lithium-ion or lithium-polymer cell - Designing an enclosure with electromagnetic interference shielding - Implementing multiple access so that systems can share available Wi-Fi channels - Testing the system with a real electric guitar and amplifier.